MILO Firmware V1.02 Release Notes

March 27, 2006

Subunit versions:

SH4 V0.38

DSP V0.00.000046

Priority FPGA V00.87

I2C_FPGA V01.16

BG_FPGA V00.20

XPOINT_FPGA V00.05

WCP Applet V1.16

Requires MILO Boot Loader V0.22, CPLD V1.05

Firmware changes:

- Label size is now a window attribute.  The WindowLabelSIZE command now requires a window number.

- Label font quality improved.

- The ForeGroundENable command now takes IMAGE, CLOCK, and CURSOR values.  CLOCK and CURSOR options are options enabled via a special command.  (Fee based option)

- The CLOCKSIZE, CLOCKForMat, and CLOCKCOLOR commands were added.

- The telnet server on port 8000 now requires login.  Default password is ‘admin’.

- The last line on each display was not being written during JPEG background loads.  This has been fixed.

- Window border properties are now imported correctly with the SETTINGSIMPORT command. (PR221)

- The WINSRCRECTREL command with a duration formerly crashed the box.  This has been fixed. (PR219)

- BGSAVE, FGSAVE, and MACROSAVE now report an error when Compact Flash is full. (PR216)

- V1.01 had boot and update problems when front panel Compact Flash wasn’t installed.  This has been fixed.  (PR222)

- The OutPutENable and OutPutID commands have been removed. (PR217, PR218)

- Window Layouts now match their depictions in the user manual. (PR207)

- A bug in the boot loader was preventing the system from booting from 128M and larger Compact Flash cards.  This has been fixed in Boot Loader V0.22.

- The PRIority command now behaves rationally. (PR125)

- Foregrounds (including cursor and clock) now correctly appear at the highest priority.

- The INputCALibration command now works.  INCAL ALL will calibrate all 6 graphics inputs.

- The ForeGroundLOAD command would hang the system when attempting to load over-sized images.  This has been fixed. (PR226)

- Window contents were showing through the foreground on the right and bottom edges of the window.  This has been fixed. (PR234)

- Serial port baud rate was getting reset to 9600 baud when upgrading from V1.00 to 1.02.  This has been fixed. (PR236)

- The WCP was reporting incorrect input types at startup.  This has been fixed. (PR231)

- Attempts were made to improve the responsiveness of the on-screen cursor.

- Telnet port 8000 login now handles backspace and delete characters correctly.

- The WCP had resizing problems on windows with very small borders.  This has been fixed (PR211).

- WCP supports digital clock settings (size, format, and color).

- Modifications to NVRAM initialization code to prevent erroneous purging.

- Foreground stream rectangles are re-initialized as part of DSP restart mechanism.

NOTE:  This firmware requires Rev. 1.20 or later MILO hardware.  Graphics cards must have V-Sync ECO (# XXXXX) and H-Sync ECO (# XXXXX) applied.

This version does not support detection of interlaced inputs.