NetLinx Module Interface Specification

for a

RGB Spectrum 4View

Display Processor

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	3

Command Interface	5

Extended Command Interface	7

String Feedback	12

Device Notes	16

Programming Notes	16

LIST OF TABLES

Table 1 – Send Command Definitions	6

Table 2 - Extended Send Command Definitions	11

Table 3 - String Feedback Definitions	16

Revision History

| ###### Date | Initials | Comments |
| --- | --- | --- |
| 07-18-2003 | CGA | Initial Release |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a RGB Spectrum 4View display processor. The 4View supports an RS-232 serial protocol. The interface was tested using version 3.02.01 of the firmware on a Model 200. The required communication settings are a baud rate of 9600, 8 data bits, 1 stop bit, no parity, and handshaking off. The cable for this device is FG#10-752-10. The wiring for this cable is as follows:

AMX 4-pin phoenix – DB9 Male

1 GND – 5 GND

2 RX – 2 RX

3 TX – 3 TX

## Overview

The COMM module translates between the standard interface described below and the 4View serial protocol. It parses the buffer for responses from the windowing system, sends strings to control the windowing system, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the string responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the NetLinx module.

4View

display processor

NetLinx

COMM Module

NetLinx

UI Module

Virtual

Device

## Implementation

To interface to the AMX RGB_Spectrum_4View_Com.axs  module, the programmer must perform the following steps:

- Define the device ID for the 4View that will be controlled.

- Define the virtual device ID that the 4View COMM module will use to communicate with the main program and User Interface. NetLinx virtual devices start with device number 33001.

- If a touch panel interface is desired, a touch panel file RGB Spectrum 4View-Platinum 640x480,Rev1.TPD and module ( RGB_Spectrum_4View_UI.axs) have been created for testing.

- The NetLinx SuperView module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the 4View to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

dvDevice		= 5001:1:0	// The 4View connected to the NetLinx on 1st RS-232 port

dvTouchPanel 	= 128:1:0	// The touch panel used for output

vdvDevice       	= 33001:1:0	// The virtual device use for communication between the

// Comm module interface and User_Interface (UI) module interface

DEFINE_VARIABLE

//Define arrays of button channels used on your own touch panel

integer nTP_BUTTONS[]={1,2,3,4,5,6,7}

DEFINE_START	// Place define_module calls to the very end of the define_start section.

// Comm module

DEFINE_MODULE ‘Device_Comm’ Comm1(dvDevice, vdvDevice)

// Touch panel module

DEFINE_MODULE 'User_Interface' TouchPanelA(vdvDevice, dvTouchPanel, nTP_BUTTONS)

Upon initialization the AMX Comm module will communicate with the 4View and information will be exchanged.

## Command Interface

The UI module controls the 4View via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

| Command | Description |
| --- | --- |
| BRIGHTNESS=<value>[:<input>] | Adjust the brightness level for the specified input. If no input is specified, brightness is set for all inputs. Note: This command is not valid for SDI inputs. <value> = 0..100 = API range -500..500 = Device specific range <input> = 1-4 = selected input BRIGHTNESS=60 BRIGHTNESS=100:3 |
| BRIGHTNESS?[<input>] | Query for the current brightness setting. If no input is specified, then all input channels are queried. Note: This command is not valid for SDI inputs. Input : 1-4 = selected input BRIGHTNESS?3 |
| CONTRAST=<value>[:<input>] | Adjust the contrast level of the specified input. If no input is specified, contrast is set for all inputs. Note: This command is not valid for SDI inputs. <value> = 0-100 = API range 0-199 = Device specific range <input> = 1-4 = selected input CONTRAST=30 CONTRAST=50:3 |
| CONTRAST?[<input>] | Query for the current contrast setting for the specified input. If no input is specified, all input channels are queried. Note: This command is not valid for SDI inputs. <input> : 1-4 = selected input CONTRAST?4 |
| DEBUG=<state> | Set the state of the telnet debugging messages. <state>: 0 = off 1 = on DEBUG=1 |
| DEBUG? | Query the state of the telnet debugging messages. DEBUG? |
| DEVICE_SCALE=<value> | Sets the number scale used by commands and strings. <state>: 0 = Use standard API range (1..100) 1 = Use hardware specific range (device specific) DEVICE_SCALE=0 |
| DEVICE_SCALE? | Queries for the number scale. DEVICE_SCALE? |
| PASSTHRU=<string> | Allows user the capability of sending commands directly to whatever unit is attached without processing by the NetLinx module. User must be aware of the protocol implemented by the unit to use this command. This gives the user access to features, which may not be directly supported by the module. Note: The Comm module appends the terminating characters to the command for you. <string> : string to send to unit PASSTHRU=THIS IS A COMMAND PASSTHRU=RESET |
| SHARPNESS=<value>[:<input>] | Adjust the sharpness level for the specified input. If no input is specified, then sharpness is set on all inputs. Note: This command is not valid for SDI inputs. <value> : 0-100 = API range -8..7 = Device specific range <input> : 1-4 = selected input SHARPNESS=52 SHARPNESS=25:3 |
| SHARPNESS?[<input>] | Query for the current sharpness setting of the specified input. If no input is specified, then all input channels are queried. Note: This command is not valid for SDI inputs. <input> : 1-4 = selected input SHARPNESS?4 |
| VERSION? | Query for the current version number of the NetLinx module. VERSION? |

Table 1 – Send Command Definitions

## Extended Command Interface

This display processor requires some “extended” commands that go beyond the standard command interface described above.  These extended commands are listed below.

| Command | Description |
| --- | --- |
| BANK=<input> | Simultaneously switch all 4 input channels between Bank A and Bank B inputs, or vice versa. Note: This command is valid for composite inputs only. <input> : A = Switch to Bank A inputs B = Switch to Bank B inputs BANK=A |
| BORDER=<state> | Set the on/off state of the quadrant and full screen borders. Note: Turning off borders in Quad mode results in no spacing between quadrants. <state> : 0 = Off 1 = On BORDER=1 |
| BORDER? | Query for the state of the quadrant/full screen borders. BORDER? |
| BORDER_COLOR=<color> | Set the color of the quadrant borders. <color> : WHITE YELLOW CYAN GREEN MAGENTA RED BLUE BLACK GRAY DARK YELLOW DARK CYAN DARK GREEN DARK MAGENTA DARK RED DARK BLUE TRANSPARENT BORDER_COLOR=BLUE |
| BORDER_SIZE=<vert>:<horiz> | Set the vertical and horizontal border size for the quadrants. <vert> : 0-99 = pixel width of vertical borders <horiz>: 1-99 = line height of horizontal borders BORDER_SIZE=5:22 |
| BORDER_SIZE? | Query the current vertical and horizontal border size for the quadrants. BORDER_SIZE? |
| DIMMER | Changes the brightness of the unit’s front panel. DIMMER |
| DIMMER? | Query for the current brightness setting of the unit’s front panel. DIMMER? |
| FIRMWARE? | Query the firmware version and model of the 4View. FIRMWARE? |
| FRAME_LOCK=<state> | Enable or disable the output frame-rate lock to the input frame-rate. Note: The input source is always input 1. <state> : 0 = Off 1 = On FRAME_LOCK=1 |
| FRAME_LOCK? | Query the state of the output frame rate lock. FRAME_LOCK? |
| FREEZE=<state>[:<input>] | Freeze or unfreeze the specified input. If no input is specified, then freeze is applied to all inputs. <state> : 0 = unfreeze 1 = freeze T = toggle (only valid when input specified) <input> : 1-4 = selected input FREEZE=1 FREEZE=0:2 |
| FREEZE?[<input>] | Query for the current freeze state for the specified input. If no input is specified, then all input channels are queried. <input> : 1-4 = selected input FREEZE?4 |
| FULL_SCREEN=<input> | Select input(s) to be displayed on the output screen. Note: Quadrant view is only valid in the Normal format. See OPERATING_STANDARD= command for more details. <input> : 0 = All inputs in quadrant view 1 = Input 1 2 = Input 2 3 = Input 3 4 = Input 4 FULL_SCREEN=0 |
| FULL_SCREEN? | Query for the current input being displayed full screen. FULL_SCREEN? |
| GLOBAL_SAVE | Force an update and explicit save of the system’s NVRAM. GLOBAL_SAVE |
| HUE=<value>[:<input>] | Adjust the hue level for the specified input. If no input is specified, then hue is set on all inputs. Note: This command is not valid for SDI inputs. <value> : 0..100 = API range -180..180 = Device specific range <input> : 1-4 = selected input HUE=52 HUE=25:3 |
| HUE?[<input>] | Query for the current hue setting of the specified input. If no input is specified, then all input channels are queried. Note: This command is not valid for SDI inputs. <input> : 1-4 = selected input HUE?4 |
| INPUT_NAME=<input>:[<name>] | Assign a label to the designated input. Note: Sending a null string in for name clears the name. <input> : 1-4 = Input source <name> : string composed of A-Z,0-9, and space INPUT_NAME=1:DVD_PLAYER INPUT_NAME=4: |
| INPUT_NAME?<input> | Query for the current label assigned to the selected input. <input> : 1-4 = Input INPUT_NAME?3 |
| INPUT_TYPE=<input>:<type> | Select the video type for the specified input. <input> : 1-4 = selected input <type> : COMPOSITE-A COMPOSITE-B S-VIDEO SDI ( only valid for model 100-SDI ) INPUT_TYPE=1:S-VIDEO |
| INPUT_TYPE?<input> | Query the type for the selected input. <input> : 1-4 = selected input INPUT_TYPE?4 |
| LCD_CONTRAST=<value> | Adjust the contrast level of the front panel LCD. <value> = 0-100 = API range 0-14 = Device specific range LCD_CONTRAST=3 |
| LCD_CONTRAST? | Query for the current contrast setting for the front panel LCD. LCD_CONTRAST? |
| LOCK=<state> | Set the on/off state of the front panel lock out. <state> : 0 = Off 1 = On LOCK=1 |
| LOCK? | Query for the current state of the front panel lock out. LOCK? |
| NAME=<state> | Set the on/off state of the input name labels. <state> : 0 = Off 1 = On NAME=1 |
| NAME? | Query for the on/off state of the input name labels. NAME? |
| NAME_COLOR=<color> | Set the color of the input name labels. <color> : WHITE YELLOW CYAN GREEN MAGENTA RED BLUE BLACK GRAY DARK YELLOW DARK CYAN DARK GREEN DARK MAGENTA DARK RED DARK BLUE TRANSPARENT NAME_COLOR=WHITE |
| NAME_COLOR? | Query the color of the input name labels. NAME_COLOR? |
| NAME_POSITION=<state> | Set the positioning of the input name labels. <state> : T = Top of image B = Bottom of image NAME_POSITION=T |
| NAME_POSITION? | Query the positioning on the input name labels. NAME_POSITION? |
| OPERATING_STANDARD=<standard> | Configures the 4View for a new operating standard. Note: This command results in reset and reconfiguration of the device. Due to the re-initalization of the device, the following response commands are also returned: DIMMER=, FRAME_LOCK=, FULL_SCREEN=, and LOCK=. <standard> : 1 = NTSC mode, Normal format, Progressive Scan 2 = NTSC mode, Normal format, Interlaced Scan 3 = NTSC mode, Wide format, Progressive Scan 4 = PAL mode, Normal format, Interlaced Scan OPERATING_STANDARD=1 |
| OPERATING_STANDARD? | Query for the current operating standard configuration of the 4View. OPERATING_STANDARD? |
| OVER_SCAN=<input>:<state> | Set the on/off state of the overscan functionality for a specific input. <input> : 1-4 = selected input <state> : OFF ON OVER_SCAN=1:ON |
| OVER_SCAN?<input> | Query the current setting of the overscan functionality for a specific input. <input> : 1-4 = selected input OVER_SCAN?4 |
| RESTORE_DEFAULTS | Restores all user settings to their factory default values. Note: This command erases all settings and labels and may take a few moments to complete. Note: This command results in reset and reconfiguration of the device. Due to the re-initalization of the device, the following response commands are returned: DIMMER=, FRAME_LOCK=, FULL_SCREEN=, and LOCK=. RESTORE_DEFAULTS |
| SATURATION=<value>[:<input>] | Adjust the saturation level for the specified input. If no input is specified, then saturation is set on all inputs. Note: This command is not valid for SDI inputs. <value> : 0-100 = API range 0-199 = Device specific range <input> : 1-4 = selected input SATURATION=52 SATURATION=25:3 |
| SATURATION?[<input>] | Query for the current saturation setting of the specified input. If no input is specified, then all input channels are queried. Note: This command is not valid for SDI inputs. <input> : 1-4 = selected input SATURATION?4 |

Table 2 - Extended Send Command Definitions

## String Feedback

The NetLinx COMM module provides feedback to the User Interface module for 4View changes via string events. The strings supported are listed below.

| String | Description |
| --- | --- |
| BANK=<input> | Feedback on the current input banks being used for composite inputs. <input> : A = Bank A inputs B = Bank B inputs BANK=A |
| BORDER=<state> | Feedback on the on/off state of the quadrant borders. <state> : 0 = Off 1 = On BORDER=1 |
| BORDER_COLOR=<color> | Feedback on the color of the quadrant borders. <color> : WHITE YELLOW CYAN GREEN MAGENTA RED BLUE BLACK GRAY DARK YELLOW DARK CYAN DARK GREEN DARK MAGENTA DARK RED DARK BLUE TRANSPARENT BORDER_COLOR=BLUE |
| BORDER_SIZE=<vert>:<horiz> | Feedback on the vertical and horizontal border size for the quadrants. <vert> : 0-99 = pixel width of vertical borders <horiz>: 1-99 = line height of horizontal borders BORDER_SIZE=5:22 |
| BRIGHTNESS=<value>:<input> | Feedback on the current brightness setting of the specified input. <value> : 0-100 = API percent range -500..500 = device specific range <input> : 1-4 = selected input BRIGHTNESS=50:3 |
| CONTRAST=<value>:<input> | Feedback on the current contrast setting of the specified input. <value> : 0-100 = API percent range 0-199 = device specific range <input> : 1-4 = selected input CONTRAST=50:3 |
| DEBUG=<state> | Feedback on the on/off state of the telnet debugging messages. <state> : 0 = Off 1 = On DEBUG=1 |
| DEVICE_SCALE=<state> | Feedback on the on/off state of device specific ranges used with commands and strings. <state> : 0 = Off 1 = On DEVICE_SCALE=0 |
| DIMMER=<state> | Feedback on the brightness of the unit’s front panel. <state> : 0 = Off 1 = On DIMMER=1 |
| ERRORM=<msg> | Feedback on an error that occurred within the Comm module or the physical device. <msg> : string ERRORM=Invalid command. |
| FIRMWARE=<version> | Returns the firmware version of the physical device. <version> : string FIRMWARE=1.1 |
| FRAME_LOCK=<state> | Feedback on the state of the output frame-rate lock to the input frame-rate. <state> : 0 = Off 1 = On FRAME_LOCK=1 |
| FREEZE=<state>:<input> | Feedback on the on/off freeze state of a specific input’s window. <state> : 0 = Off 1 = On <input> : 1-4 = selected input FREEZE=1:2 |
| FULL_SCREEN=<input> | Feedback on which input is being displayed full screen. <input> : 0 = No input in full screen (device in quad view) 1 = Input 1 2 = Input 2 3 = Input 3 4 = Input 4 FULL_SCREEN=1 |
| HUE=<input>:<value> | Reports the current hue setting for a specific input source. <input> : 1-4 = input source <value> : 0-100 = API percent range -180..180 = device specific range HUE=2:50 |
| INPUT_NAME=<input>:<name> | Feedback on the label assigned to the designated input. Note: If name is empty, then the name has been cleared. <input> : 1-4 = Input source <name> : string composed of A-Z,0-9, and space INPUT_NAME=1:DVD_PLAYER INPUT_NAME=4: |
| INPUT_TYPE=<input>:<type> | Feedback on the video type selected for the specified input. <input> : 1-4 = selected input <type> : COMPOSITE-A COMPOSITE-B S-VIDEO INPUT_TYPE=4:S-VIDEO |
| LCD_CONTRAST=<value> | Feedback on the current contrast setting of the front panel LCD. <value> : 0-100 = API percent range 0-14 = device specific range LCD_CONTRAST=5 |
| LOCK=<state> | Feedback on the on/off state of the front panel lock out. <state> : 0 = Off 1 = On LOCK=1 |
| MODEL=<model> | Feedback on the model number of this device. MODEL=4View-200 |
| NAME=<state> | Feedback on the on/off state of the input name labels. <state> : 0 = Off 1 = On NAME=1 |
| NAME_COLOR=<color> | Feedback on the color of the input name labels. <color> : WHITE YELLOW CYAN GREEN MAGENTA RED BLUE BLACK GRAY DARK YELLOW DARK CYAN DARK GREEN DARK MAGENTA DARK RED DARK BLUE TRANSPARENT NAME_COLOR=WHITE |
| NAME_POSITION=<state> | Feedback on the positioning of the input name labels. <state> : T = Top of image B = Bottom of image NAME_POSITION=T |
| OPERATING_STANDARD=<mode>:<format>:<scan> | Feedback on the current operating standard of the device. <mode> : N = NTSC mode P = PAL mode <format> : N = Normal format W = Wide format <scan> : P = Progressive Scan I = Interlaced Scan OPERATING_STANDARD=N:N:P |
| OVER_SCAN=<input>:<state> | Feedback on the on/off state of the overscan functionality for the specified input. <input> : 1-4 = selected input <state> : OFF ON OVER_SCAN=1:ON |
| SATURATION=<input>:<value> | Returns the saturation value as a percent for the specified input source. <input> : 1-4 = input source <value> : 0-100 = API percent range 0-199 = device specific range SATURATION=4:50 |
| SHARPNESS=<value>:<input> | Feedback on the current sharpness setting of the specified input. <value> : 0-100 = API percent range -8..7 = device specific range <input> : 1-4 = selected input SHARPNESS=50:3 |
| VERSION=<value> | Reports the current version number of the NetLinx module. <value> : current version number in xx.yy format VERSION=1.06 |

Table 3 - String Feedback Definitions

## Device Notes

## Programming Notes