NetLinx Module Interface Specification

for a

RGB Spectrum DualView XL

Video Windowing System

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	3

Command Interface	5

String Feedback	20

Protocol Issues	26

Device Notes	26

Programming Notes	26

Adding Functions to Modules	26

Commands to the device	26

Additional Feedback from the device	27

LIST OF TABLES

Table 1 – Send Command Definitions	19

Table 2 - String Feedback Definitions	26

Revision History

| ###### Date | Initials | Comments |
| --- | --- | --- |
| 06-01-2004 | CGA | Initial Release |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a RGB Spectrum DualView XL video windowing system. The DualView XL supports an RS-232 protocol over a serial connection. The interface was tested using version 1.04 of the firmware. The required serial communication settings are a baud rate of 9600, 8 data bits, 1 stop bit, no parity, and handshaking off. The serial cable for this device is FG#10-752. The wiring for this cable is as follows:

AMX – DB9 Male

GND – 5 GND

RX – 2 RX

TX – 3 TX

## Overview

The COMM module translates between the standard interface described below and the DualView XL serial protocol. It parses the buffer for responses from the windowing system, sends strings to control the windowing system, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the string responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the NetLinx module.

DualView XL

NetLinx

COMM Module

NetLinx

UI Module

Virtual

Device

## Implementation

To interface to the AMX RGB_Spectrum_DualView_XL_Com.tko module, the programmer must perform the following steps:

- Define the device ID for the DualView XL that will be controlled.

- Define the virtual device ID that the DualView XL COMM module will use to communicate with the main program and User Interface. NetLinx virtual devices start with device number 33001.

- If a touch panel interface is desired, a touch panel file RGB Spectrum DualView XL-Platinum 640x480,Rev1.TPD and module ( RGB_Spectrum_DualView_XL_UI.axs) have been created for testing.

- The NetLinx DualView XL module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the DualView XL to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

dvDevice		= 5001:1:0	// The DualView XL connected to the NetLinx on 1st RS-232 port

dvTouchPanel 	= 128:1:0	// The touch panel used for output

vdvDevice       	= 33001:1:0	// The virtual device use for communication between the

// Comm module interface and User_Interface (UI) module interface

DEFINE_VARIABLE

//Define arrays of button channels used on your own touch panel

integer nCHAN_BTN[]={1,2,3,4,5,6,7}

integer nTXT_BTN[]={1,2,3}

DEFINE_START	// Place define_module calls to the very end of the define_start section.

// Comm module

DEFINE_MODULE RGB_Spectrum_DualView_XL_Comm’ Comm1(vdvDevice, dvDevice)

// Touch panel module

DEFINE_MODULE 'RGB_Spectrum_DualView_XL_UI' TP(vdvDevice, dvTouchPanel, nCHAN_BTN, nTXT_BTN)

Upon initialization the AMX Comm module will communicate with the DualView XL and information will be exchanged.

## Command Interface

The UI module controls the DualView XL via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

Note: * denotes commands that are extensions beyond the standard API.

| Command | Description |
| --- | --- |
| ASPECT=<state>[:<input>] | Sets the current aspect ratio for the specified input source. If no input source is given, all input sources are set. Note: This command is only valid with video inputs. <state> : 1 = Normal (1.33:1) 2 = Widescreen1 (1.66:1) 3 = Widescreen2 (16:9) 4 = 4:3 – Not Supported 5 = Widescreen3 (1.85:1) 6 = Widescreen4 (2.35:1) <input> : 1-2 = input source ASPECT=1:2 |
| ASPECT?[<input>] | Query for the current aspect ratio for the specified input. If no input source is specified, then all input sources are queried. Note: This command is only valid with video inputs. <input> : 1-2 = specific input source ASPECT?2 |
| *BACKGROUND=[<color>] [<red> :<green> :<blue>] | Sets the color of the background. The color can be set by using the <red>, <green>, <blue> values, or by using one of the pre-defined <color> values. Note: This command returns only RGB values. <color> : BLACK BLUE BLUERAMP = blue vertical shading CYAN GREEN GREENRAMP = green vertical shading HBLUERAMP = blue horizontal shading HGREENRAMP = green horizontal shading HREDRAMP = red horizontal shading HWHITERAMP = white horizontal shading MAGENTA RED REDRAMP = red vertical shading WHITE WHITERAMP = white vertical shading YELLOW <red> : 0-255 = red color value <green> : 0-255 = green color value <blue> : 0-255 = blue color value BACKGROUND=HWHITERAMP BACKGROUND=150:0:100 |
| *BACKGROUND? | Query for the current background color. Note: This query returns only RGB values. BACKGROUND? |
| *BAUD=<rate> | Sets the serial port baud rate of the device and the NetLinx. Default value is 9600. Note: Please see the Device Notes section. Note: This command does not have any feedback. Note: This command deletes all queued commands and executes immediately. Note: This command takes a few moments to complete. Other commands sent during this time may not be processed. <rate> : 9600 19200 38400 BAUD=9600 |
| *BAUD? | Query the current baud rate. BAUD? |
| *BORDER_COLOR=<window> [:<color>] [:<red> :<green> :<blue>] | Sets the color of the border around the specified window. The color can be set by using the <red>, <green>, <blue> values, or by using one of the pre-defined <color> values. <window> : 0 = All windows 1-2 = specific window <color> : BLACK BLUE CYAN GREEN MAGENTA RED WHITE YELLOW <red> : 0-255 = red color value <green> : 0-255 = green color value <blue> : 0-255 = blue color value BORDER_COLOR=2:WHITE BORDER_COLOR=0:150:0:100 |
| *BORDER_COLOR?[<window>] | Query for the color of the border around a specific window, or all windows. If no window is specified then all windows are queried. Note: This query returns only the RGB color values. Note: If all windows are queried, then 2 responses will be sent. <window> : 1-2 = specific window BORDER_COLOR? BORDER_COLOR?2 |
| *BORDER_WIDTH=<window>:<width> | Sets the border width around the specified window. <window> : 0 = All windows 1-2 = Specific window <width> : 0 = No border 1-100 = Border width in pixels BORDER_WIDTH=0:0 |
| *BORDER_WIDTH?[<window>] | Query for the current boarder width of a specific window. If no window is specified, then all window widths are queried and a response for each window is returned. <window> : 1-2 = Specific window BORDER_WIDTH? BORDER_WIDTH?1 |
| BRIGHTNESS=<value>[:<input>] | Adjust the brightness level for the specified input. If no input is specified, brightness is set for all inputs. <value> = 0..100 = API brightness range, in percent (%) -500..500 = Device specific range <input> = 1-2 = selected input BRIGHTNESS=60 BRIGHTNESS=100:1 |
| BRIGHTNESS?[<input>] | Query for the current brightness setting. If no input is specified, then all input channels are queried. Input : 1-2 = selected input BRIGHTNESS?2 |
| COLOR=<value>[:<input>] | Adjust the color level for the specified input. If no input is specified, then color is set on all inputs. <value> : 0-100 = API color range, in percent (%) 0-200 = Device specific range <input> : 1-2 = selected input COLOR=52 COLOR=25:2 |
| COLOR?[<input>] | Query for the current color setting of the specified input. If no input is specified, then all input channels are queried. <input> : 1-2 = selected input COLOR? COLOR?2 |
| CONTRAST=<value>[:<input>] | Adjust the contrast level of the specified input. If no input is specified, contrast is set for all inputs. <value> = 0-100 = API contrast range, in percent (%) 0-200 = Device specific range <input> = 1-2 = selected input CONTRAST=30 CONTRAST=50:2 |
| CONTRAST?[<input>] | Query for the current contrast setting for the specified input. If no input is specified, all input channels are queried. <input> : 1-2 = selected input CONTRAST?2 |
| DEBUG=<state> | Set the state of the telnet debugging messages. <state>: 0 = off 1 = on DEBUG=1 |
| DEBUG? | Query the state of the telnet debugging messages. DEBUG? |
| DEVICESCALE=<value> | Sets the number scale used by commands and strings. <state>: 0 = Use standard API range (1..100) 1 = Use hardware specific range (device specific) DEVICESCALE=0 |
| DEVICESCALE? | Queries for the number scale. DEVICESCALE? |
| DUALVIEW=<style> | Switches the output display to a side-by-side configuration, using the input arrangement specified in the Map= command. <style> : 1 = Crop 2 = Normal 3 = Stretch DUALVIEW=2 |
| DUALVIEW? | Query for the style currently being used by the mapping function. DUALVIEW? |
| *FIRMWARE? | Query the firmware version of the DualView XL. FIRMWARE? |
| *FREEZE=<state>[:<input>] | Freeze or unfreeze the specified input. If no input is specified, then freeze is applied to all inputs. <state> : 0 = unfreeze 1 = freeze T = toggle (only valid when input specified) <input> : 1-2 = selected input FREEZE=1 FREEZE=T:2 |
| *FREEZE?[<input>] | Query for the current freeze state for the specified input. If no input is specified, then all input channels are queried. <input> : 1-2 = selected input FREEZE?2 |
| *FULL_SCREEN=<input> | Set the selected window to a full screen display. Note: This command has no feedback. <input> : 0 = No input channel 1-2 = input channel FULL_SCREEN=0 |
| *GAMMA=<value> | Adjust the gamma value for the device. <value> : 0-100 = API gamma range, in percent (%) 0.5-2.0 = Device specific range GAMMA=52 |
| *GAMMA? | Query for the current gamma setting for the device. GAMMA? |
| *GLOBAL_SAVE | Force an update and explicit save of the system’s NVRAM. Note: This command has no feedback. GLOBAL_SAVE |
| *HUE=<value>[:<input>] | Adjust the hue level for the specified input. If no input is specified, then hue is set on all inputs. <value> : 0..100 = API hue range, in percent (%) -180..180 = Device specific range <input> : 1-2 = selected input HUE=52 HUE=25:2 |
| *HUE?[<input>] | Query for the current hue setting of the specified input. If no input is specified, then all input channels are queried. <input> : 1-2 = selected input HUE?2 |
| *INITIALIZE | Properly configures communication between the NetLinx and the device. Both devices must be online and connected. Note: This is only necessary if both devices are online and connected, but do not appear to be talking to each other. Note: This command clears all commands currently queued to be sent to the device. Note: This command does not have any feedback. INITIALIZE |
| INPUT=<source>[:<input>] | Select the video source. If no input is specified, then all inputs are set to the specified source <source> : 1 = RGB 2 = S-VIDEO 3 = DVI 4 = COMPOSITE1 5 = COMPONENT – Not Supported 6 = COMPOSITE2 7 = YUV <input> : 1-2 = selected input INPUT =1:2 INPUT =2 |
| INPUT?[<input>] | Query the input source. If no input is specified, then all inputs are queried. <input> : 1-2 = selected input INPUT?2 |
| *INPUT_LIST_LOAD=<input> :<entry> | Loads the indicated entry from the Input List to the specified input channel. Note: The entry is only loaded if it matches the measured signal parameters – sync format and polarity, interlace state, vertical total, and horizontal frequency. Note: This command has no feedback. <input> : 1-2 = input channel <entry> : 1-160 = Input List entry INPUT_LIST_LOAD=2:140 |
| *INPUT_LIST_SAVE=<input> :<entry> | Save the specified input to the selected entry in the Input List. Note: This command is for RGB input sources only. Note: This command has no feedback. <input> : 1-2 = specified input source <entry> : 100-160 = place in Input List INPUT_LIST_SAVE=1:2 |
| *INPUT_MODE=<input>:<state> | Set the input mode for the specified input. <input> : 0 = All input channels 1-2 = Specific input channel <state> : AUTO = Engages autosync circuitry LOCK = Disengages autosync circuitry DEBUG = Provides status INPUT_MODE=1:AUTO |
| *INPUT_MODE?[<input>] | Query the mode for the specified input. If no channel is specified, then all input channels are queried. <input> : 1-2 = selected input channel INPUT_MODE?2 |
| *INPUT_NAME=<input>:<name> | Set the name for a specified input with a locked signal. Note: The name may not contain any spaces, but it may contain underscores. <input> : 1-2 = specific input source <name> : string of up to 23 alphanumeric characters INPUT_NAME=1:DVD_Player |
| *INPUT_NAME?[<input>] | Query for the assigned name of a specified input source, or all sources. If no input is specified, then all input sources are queried. <input> : 1-2 = specific input source INPUT_NAME?1 |
| *INPUT_PHASE=<input>:<value> | Adjusts the phase of the Analog/Digital Converter (ADC sample clock phase) to optimize picture quality. <input> : 1-2 = input source <value> : 0-31 = phase setting INPUT_PHASE=2:30 |
| *INPUT_PHASE?<input> | Query for the phase of the Analog/Digital Converter (ADC sample clock phase). <input> : 1-2 = input source INPUT_PHASE?2 |
| *INPUT_TIMING=<input> :<mode> :<direction> | Enters input interactive mode to visually adjust timing parameters of the specified input. Modes LR and UL require the optional direction parameter. Note: This command has no feedback. <input> : 1-2 = specified input channel <mode> : LR = adjust lower right corner UL = adjust upper left corner <direction> : + = move corner up - = move corner down < = move corner left > = move corner right INPUT_TIMING=1:LR:+ INPUT_TIMING=1:UL:< |
| *INPUT_WINDOW=<input> :<x> :<y> :<width> :<height> | Set the input window (input source rectangle) for a specific input. Note: Video inputs are limited to 700x480. RGB inputs are limited to 1280x1024. Note: If an unacceptable value is sent for a particular input type, then the device will change the invalid value to the largest acceptable value for that input type. <input> : 1-2 = specific input source <x> : 0-1920 = X-coordinate for top left corner of input window <y> : 0-1920 = Y-coordinate for top left corner of input window <width> : 2-1920 = pixel width of input window <height>: 2-1920 = pixel height of input window INPUT_WINDOW=1:0:0:400:300 |
| *INPUT_WINDOW?<input> | Query for the current dimensions of the specified input window (input source rectangle). <input> : 1-2 = specific input source INPUT_WINDOW?1 |
| *LABEL_BGCOLOR=<window> [:<color>] [:<red> :<green> :<blue>] | Sets the color of the background behind the label text for the specified window label. The color can be set by using the <red>, <green>, <blue> values, or by using one of the pre-defined <color> values. Note: This command returns only RGB values. <window> : 0 = All windows 1-2 = specific window <color> : BLACK BLUE CYAN GREEN MAGENTA RED WHITE YELLOW <red> : 0-255 = red color value <green> : 0-255 = green color value <blue> : 0-255 = blue color value LABEL_BGCOLOR=2:WHITE LABEL_BGCOLOR=0:150:0:100 |
| *LABEL_BGCOLOR?[<window>] | Query for the color of the background behind the label text on a specified window label. If no window is specified then all windows are queried. Note: This query returns only the RGB color values. Note: If all windows are queried, then 4 responses will be sent. <window> : 1-2 = specific window LABEL_BGCOLOR? LABEL_BGCOLOR?2 |
| *LABEL_COLOR=<window> [:<color>] [:<red> :<green> :<blue>] | Sets the color of the text for the specified window label. The color can be set by using the <red>, <green>, <blue> values, or by using one of the pre-defined <color> values. <window> : 0 = All windows 1-2 = specific window <color> : BLACK BLUE CYAN GREEN MAGENTA RED WHITE YELLOW <red> : 0-255 = red color value <green> : 0-255 = green color value <blue> : 0-255 = blue color value LABEL_COLOR=2:WHITE LABEL_COLOR=0:150:0:100 |
| *LABEL_COLOR?[<window>] | Query for the color of the text for the specified window label. If no window is specified then all windows are queried. Note: This query returns only the RGB color values. Note: If all windows are queried, then 4 responses will be sent. <window> : 1-2 = specific window LABEL_COLOR? LABEL_COLOR?2 |
| *LABEL_ENABLE=<window>:<state> | Turns on or off the specified window’s label. <window> : 0 = All windows 1-2 = Specific window <state> : 0 = Off 1 = On LABEL_ENABLE=1:1 LABEL_ENABLE=0:0 |
| *LABEL_ENABLE?[<window>] | Query for the on/off state of a specific window’s label. If no window is specified, then all windows are queried and a response is returned for each. <window> : 1-2 = Specific window LABEL_ENABLE? LABEL_ENABLE?2 |
| *MAC_ADDR? | Query for the current Ethernet MAC address. MAC_ADDR? |
| *MAP=<input1> :<input2> | Provides the ability to assign inputs to different sides of the display without having to reposition windows. Note: Each input can only be assigned to one window. <input1> : 1-2 = input for window 1 <input2> : 1-2 = input for window 2 MAP=2:1 |
| *MAP? | Query for the current window assignments for ‘DUALVIEW’ display. MAP? |
| *MOVE=<input>:<direction> | Move a specified input’s window around the output display. Note: The currently active window must be disabled before another window can be enabled. Note: This command has no feedback. <input> : 1-2 = selected input <direction> : > = move right < = move left + = move up - = move down MOVE=2:< MOVE=2:S |
| *OUTPUT_FADE=<state>[:<time>] | Enables or disables the output signal. When used with the optional time parameter, the output will fade to black for the specified duration. <state> : 0 = Off 1 = On <time> : 1-50 = time in tenths of seconds OUTPUT_FADE=1:25 OUTPUT_FADE=0:10 |
| *OUTPUT_FADE? | Query for the current state of the output signal. Note: This query does not return the time parameter. OUTPUT_FADE? |
| *OUTPUT_LIST_LOAD=<entry> | Sets the output display parameters from the values stored in the specified Timing List entry. Note: This command has no feedback. <entry> : 1-160 = Input List entry OUTPUT_LIST_LOAD=140 |
| *OUTPUT_LIST_NAME=<name> | Sets the name of the Timing List entry currently in use by the output. Note: The name may not contain any spaces, but it may contain underscores. <name> : string of up to 23 alphanumeric characters OUPUT_LIST_NAME=Monitor |
| *OUTPUT_LIST_NAME? | Query the name of the current Timing List entry used by the output. OUTPUT_LIST_NAME? |
| *OUTPUT_LIST_SAVE=<place> | Save the current output source settings into the Timing List. Note: This command has no feedback. <place> : 100-160 = place in the Timing List OUTPUT_LIST_SAVE=104 |
| *OUTPUT_MODE=<mode> | Set the output’s timing reference. <mode> : DEBUG = Set debug reference EDID = Set to EDID reference EXT = Lock to input 1’s input signal INT = Free run mode OUTPUT_MODE=INT |
| *OUTPUT_MODE? | Query the mode for the output. OUTPUT_MODE? |
| *OUTPUT_TIMING=<mode> :<direction> | Enters output interactive mode to visually adjust timing parameters of the output device. Note: This command has no feedback. <mode> LR = adjust lower right corner UL = adjust upper left corner <direction> : + = move corner up - = move corner down < = move corner left > = move corner right OUTPUT_TIMING=LR:+ OUTPUT_TIMING=UL:< |
| *OUTPUT_WINDOW=<input> :<x> :<y> :<width> :<height> | Set the output window (input’s destination rectangle) for a specific input source. <input> : 1-2 = specific input source <x> : 0-n = X-coordinate for top left corner of \ output window <y> : 0-n = Y-coordinate for top left corner of output window <width> : 0-1920 = pixel width of output window <height>: 0-1440 = pixel height of output window OUTPUT_WINDOW=2:0:0:1024:768 |
| *OUTPUT_WINDOW?<input> | Query the current dimensions of the output window (input’s destination rectangle) for a specific input source. <input> : 1-2 = input source OUTPUT_WINDOW?1 |
| *OVER_SCAN=<input>:<state> | Set the on/off state of the overscan functionality for a specific input. Note: This command is only valid for s-video and composite inputs. <input> : 1-2 = selected input <state> : OFF ON OVER_SCAN=1:ON |
| *OVER_SCAN?<input> | Query the current setting of the overscan functionality for a specific input. Note: This command is only valid for s-video and composite inputs. <input> : 1-2 = selected input OVER_SCAN?2 |
| *PANTILT=<input>:<value> | Move the input’s view up, down, left, and right. Note: This command has no feedback. <input> : 1-2 = selected input <value> : 1 = step tilt down - = step pan right - = step tilt up 7 = step pan left PANTILT=2:5 |
| PASSTHRU=<string> | Allows user the capability of sending commands directly to whatever unit is attached without processing by the NetLinx module. User must be aware of the protocol implemented by the unit to use this command. This gives the user access to features, which may not be directly supported by the module. For more information, see the “Adding Functions to Modules” section. Note: The required carriage return is automatically appended to all passthru commands. <string> : string to send to unit PASSTHRU=THIS IS A COMMAND PASSTHRU=RESET |
| *PRESET_DEL=<entry> | Clears the specified preset. Note: This command has no feedback. <entry> : 1-10 = preset entry PRESET_DEL=2 |
| *PRESET_LIST?<entry> | Query for details saved in a specific preset. Note: Each entry returns two responses, one for each window. <entry> : 1-10 = preset entry PRESET_LIST?4 |
| *PRESET_NAME=<entry>:<name> | Assigns a name to the specified preset. Note: Names may be up to 23 characters in length and may not contain spaces. <entry> : 1-10 = preset entry number <name> : string = preset name PRESET_NAME=5:Theater |
| *PRESET_NAME?<entry> | Query for the name of a specific preset. <entry> : 1-10 = preset entry number PRESET_NAME?6 |
| *PRIORITY=<input> :<value> | Set the priority of the selected input’s window. <input> : 1-2 = selected input <value> : 1-2 = priority value PRIORITY=2:1 |
| *PRIORITY?[<input>] | Query the current priority of the specified input channel. If no input is specified, then all input channels are queried. <input> : 1-2 = selected input PRIORITY? PRIORITY?2 |
| *RECALL=<value>[:<ALL>] | Recall a preset. By default, only the position, size, and priority level of each input window is recalled. With the optional ALL parameter, additional parameters such as input type, image controls, etc. are recalled. Note: The optional ALL parameter requires more time since more parameters are being recalled. Note: This command has no feedback. <value> : 1..10 = preset number to recall RECALL=4 RECALL=6:ALL |
| *RESET_INPUT_WINDOW=<input> | Resets the specified input window (input source rectangle) to its default values. Note: This command has no feedback. <input> : 1-2 = specific input source RESET_INPUT_WINDOW=1 |
| *RESTORE_DEFAULTS | Restores all user settings to their factory default values. Note: Rebooting takes approximately 60 seconds to complete. Note: This command has no feedback. RESTORE_DEFAULTS |
| *SAVE=<value> | Save a preset. Note: This command has no feedback. <value> : 1..10 = preset number to save to SAVE=6 |
| *SET_MOVE=<x>:<y> | Set the number of pixels and lines a window will move when the MOVE= command is used. <x> : 1-50 = number of pixels per left or right move <y> : 1-50 = number of lines per up or down move SET_MOVE=25:25 |
| *SET_MOVE? | Query the current move settings for the MOVE= command. SET_MOVE? |
| SHARPNESS=<value>[:<input>] | Adjust the sharpness level for the specified input. If no input is specified, then sharpness is set on all inputs. <value> : 0-100 = API sharpness range, in percent (%) -5..5 = Device specific range <input> : 1-2 = selected input SHARPNESS=52 SHARPNESS=25:2 |
| SHARPNESS?[<input>] | Query for the current sharpness setting of the specified input. If no input is specified, then all input channels are queried. <input> : 1-4 = selected input SHARPNESS?4 |
| *SIZE=<input>:<direction> | Allows the specified input’s window to be resized. Note: This command does not have feedback. <input> : 1-2 = selected input <direction> : + = make larger - = make smaller SIZE=2:+ |
| *TEST_PATTERN=<state> | Set the test pattern to the designated option. <state> : 0 = Off 1 = Bars 2 = Horizontal Bars 3 = Grid 4 = Ramps 5 = DVI 6 = Snow 7 = Random TEST_PATTERN=3 |
| *TEST_PATTERN? | Query the state of the Test Pattern option. TEST_PATTERN? |
| *TIMING_LIST?[<range>] | Displays the entire Input/Output Timing List, or a portion of the List if parameters are supplied. If no range is specified, then all entries are returned. Note: One response will be returned for each entry. <range> : s-e = range of entries to display, where s = start entry and e = end entry TIMING_LIST? TIMING_LIST?11-20 |
| *TIMING_LIST_CLEAR | Deletes all user-defined timing sets. This does not affect the factory specified timing entries, 1-99. Note: This command has no feedback. TIMING_LIST_CLEAR |
| *TIMING_LIST_DEL=<entry> | Deletes the specified Timing List entry. Timing List entries 1-99 are reserved for factory specified timing and may not be deleted. Note: This command has no feedback. <entry> : 100-160 = Entry TIMING_LIST_DEL=104 |
| *UNDER_SCAN=<input>:<state> | Turns on or off the underscan feature for the specified window. Note: This command is only valid for s-video and composite inputs. <input> : 1-2 = selected input <state> : OFF ON UNDER_SCAN=1:ON |
| *UNDER_SCAN?<input> | Query the current setting of the underscan functionality for a specific input. Note: This command is only valid for s-video and composite inputs. <input> : 1-2 = selected input UNDER_SCAN?2 |
| VERSION? | Query for the current version number of the NetLinx module. VERSION? |
| *VIDEO=<input> :<state> [:<time>] | Turn the video window for the specified input on or off. Note: The time parameter provides an optional fade time. <input> : 0 = all inputs 1-2 = selected input <state> : 0 = off - = on <time> : 5-50 = fade time in tenths of seconds VIDEO=2:1 VIDEO=2:0:50 |
| *VIDEO?[<input>] | Query the on/off state of the specified input’s window. If no input is specified, then all inputs are queried. Note: This query does not return the time parameter. <input> : 1-2 = selected input VIDEO? VIDEO?2 |
| VIDEOMODE?[<input>] | Query for the current video format of the specified video input source. If no input source is provided, then all input sources are queried. <input> : 1-2 = input to query VIDEOMODE?2 |
| *VIRTUAL_WINDOW=<x>:<y> | Set how far the output window can be moved out of the output screen. <x> : 640-3200 = X direction <y> : 640-3200 = Y direction VIRTUAL_WINDOW=2000:3000 |
| *VIRTUAL_WINDOW? | Query the current setting of the virtual window multipliers. VIRTUAL_WINDOW? |
| *WINDOW?<window> | Causes the specified window to jiggle, thus allowing identification of which side it is in. Note: This command does not have feedback. <window> : 1-2 = specific window WINDOW?3 |
| *ZOOM=<input>:<value> | Zoom input in (near/tele) or out (far/wide). Note: This command does not have feedback. <input> : 1-2 = input to affect <value> : + = step zoom in (tele) - = step zoom out (wide) ZOOM=1:+ |

Table 1 – Send Command Definitions

## String Feedback

The NetLinx COMM module provides feedback to the User Interface module for DualView XL changes via string events. The strings supported are listed below.

| String | Description |
| --- | --- |
| ASPECT=<state>:<input> | Feedback on the current aspect ratio for the specified input source. <state> : 1 = Normal (1.33:1) 2 = Widescreen1 (1.66:1) 3 = Widescreen2 (16:9) 4 = 4:3 – Not Supported 5 = Widescreen3 (1.85:1) 6 = Widescreen4 (2.35:1) <input> : 1-2 = specified input source ASPECT=1:2 |
| BACKGROUND=<red> :<green> :<blue> | Feedback on the color of the background. <red> : 0-255 = red color value <green> : 0-255 = green color value <blue> : 0-255 = blue color value BACKGROUND=150:0:100 |
| BAUD=<value> | Feedback on the current baud rate at which communication between the NetLinx and the DualView XL is taking place. <value> : 9600 19200 38400 BAUD=9600 |
| BORDER_COLOR=<window> :<red> :<green> :<blue> | Feedback on the color of the border around the specified window. <window> : 1-2 = specific window <red> : 0-255 = red color value <green> : 0-255 = green color value <blue> : 0-255 = blue color value BORDER_COLOR=0:150:0:100 |
| BORDER_WIDTH=<window>:<width> | Feedback on the border width around the specified window. <window> : 1-2 = Specific window <width> : 0 = No border 1-100 = Border width in pixels BORDER_WIDTH=2:0 |
| BRIGHTNESS=<value>:<input> | Feedback on the current brightness setting of the specified input. <value> : 0-100 = API brightness range, in percent (%) -500..500 = device specific range <input> : 1-2 = selected input BRIGHTNESS=50:2 |
| COLOR=<value>:<input> | Returns the color value as a percent for the specified input source. <value> : 0-100 = API color range, in percent (%) 0-200 = device specific range <input> : 1-2 = input source COLOR=50:2 |
| CONTRAST=<value>:<input> | Feedback on the current contrast setting of the specified input. <value> : 0-100 = API contrast range, in percent (%) 0-200 = device specific range <input> : 1-2 = selected input CONTRAST=50:2 |
| DEBUG=<state> | Feedback on the on/off state of the telnet debugging messages. <state> : 0 = Off 1 = On DEBUG=1 |
| DEVICESCALE=<state> | Feedback on the on/off state of device specific ranges used with commands and strings. <state> : 0 = Off 1 = On DEVICESCALE=0 |
| DUALVIEW=<style> | Feedback on the output display to a side-by-side configuration, using the input arrangement specified in the Map= command. <style> : 1 = Crop 2 = Normal 3 = Stretch DUALVIEW=2 |
| ERRORM=<msg> | Feedback on an error that occurred within the Comm module or the physical device. <msg> : string ERRORM=Invalid command. |
| FIRMWARE=<version> | Returns the firmware version of the physical device. <version> : string FIRMWARE=1.1 |
| FREEZE=<state>:<input> | Feedback on the on/off freeze state of a specific input’s window. <state> : 0 = Off 1 = On <input> : 1-2 = selected input FREEZE=1:2 |
| GAMMA=<value> | Reports the current gamma setting for the device. <value> : 0-100 = API gamma range, in percent (%) 0.5-2.0 = device specific range GAMMA=50 |
| HUE=<input>:<value> | Reports the current hue setting for a specific input source. <input> : 1-2 = input source <value> : 0-100 = API hue range, in percent (%) -180..180 = device specific range HUE=2:50 |
| INPUT=<source>:<input> | Feedback on the video type selected for the specified input. <source> : 1 = RGB 2 = S-VIDEO 3 = DVI 4 = COMPOSITE1 5 = COMPONENT – Not Supported 6 = COMPOSITE2 7 = YUV <input> : 1-2 = selected input INPUT=2:2 |
| INPUT_MODE=<input>:<state> | Feedback on the current input mode of the specified input. <input> : 1-2 = selected input <state> : AUTO = autosync circuitry is engaged LOCK = autosync circuitry is disengaged DEBUG = providing status INPUT_MODE=2:AUTO |
| INPUT_NAME=<input>:<name> | Feedback on the name for a specified input with a locked signal. <input> : 1-2 = specific input source <name> : string of up to 23 alphanumeric characters INPUT_NAME=1:DVD_Player |
| INPUT_PHASE=<input>:<value> | Feedback on the phase setting of the Analog/Digital Converter (ADC sample clock phase). <input> : 1-2 = input source <value> : 0-31 = phase setting INPUT_PHASE=2:30 |
| INPUT_WINDOW=<input> :<x> :<y> :<width> :<height> | Feedback on the input window (input source rectangle) for the specified input. <input> : 1-2 = specified input source <x> : 0-1920 = X-coordinate for top left corner of input window <y> : 0-1920 = Y-coordinate for top left corner of input window <width> : 2-1920 = pixel width of input window <height>: 2-1920 = pixel height of input window INPUT_WINDOW=1:0:0:400:300 |
| LABEL_BGCOLOR=<window> :<red> :<green> :<blue> | Feedback on the color of the background behind the label text for the specified window label. <window> : 1-2 = specific window <red> : 0-255 = red color value <green> : 0-255 = green color value <blue> : 0-255 = blue color value LABEL_BGCOLOR=1:150:0:100 |
| LABEL_COLOR=<window> :<red> :<green> :<blue> | Feedback on the color of the label text for the specified window label. <window> : 1-2 = specific window <red> : 0-255 = red color value <green> : 0-255 = green color value <blue> : 0-255 = blue color value LABEL_COLOR=1:150:0:100 |
| LABEL_ENABLE=<window>:<state> | Feedback on the on/off state of the specified window’s label. <window> : 1-2 = Specific window <state> : 0 = Off 1 = On LABEL_ENABLE=1:1 LABEL_ENABLE=2:0 |
| MAC_ADDR=<value> | Feedback on the mac address of the DualView XL. <value> : xx:xx:xx:xx:xx:xx = mac address MAC_ADDR=00:50:c2:24:31:73 |
| MAP=<input1> :<input2> | Feedback on the assign inputs of each window. <input1> : 1-2 = input for window 1 <input2> : 1-2 = input for window 2 MAP=2:1 |
| OUTPUT_FADE=<state> | Feedback on the enabled/disabled state of the output signal. Note: The time parameter is never returned. <state> : 0 = Off 1 = On OUTPUT_FADE=0 |
| OUTPUT_LIST_NAME=<name> | Feedback on the name set for the current output source. <name> : string of up to 23 alphanumeric characters OUPUT_LIST_NAME=Monitor |
| OUTPUT_MODE=<state> | Feedback on the currently set output mode. <state> : DEBUG = Providing status EDID = EDID timing reference EXT = Locked to Input 1’s input signal INT = Free run mode OUTPUT_MODE=INT |
| OUTPUT_WINDOW=<input> :<x> :<y> :<width> :<height> | Feedback on the output window (input’s destination rectangle) for a specific input source. <input> : 1-2 = specific input source <x> : 0-n = X-coordinate for top left corner of output window <y> : 0-n = Y-coordinate for top left corner of output window <width> : 0-1920 = pixel width of output window <height>: 0-1440 = pixel height of output window OUTPUT_WINDOW=2:0:0:1024:768 |
| OVER_SCAN=<input>:<state> | Feedback on the on/off state of the overscan functionality for the specified input. <input> : 1-2 = selected input <state> : OFF ON OVER_SCAN=1:ON |
| PRESET_LIST=<entry> :<name> :<window> :<srcx> :<srcy> :<srcw> :<srch> :<dstx> :<dsty> :<dstw> :<dsth> :<bdr> :<label> | Feedback on the details saved in a specific preset. <entry> : 1-10 = preset entry <name> : string = preset name <window> : 1-2 = window <srcx> : 0-n = source window x-coordinate <srcy> : 0-n = source window y-coordinate <srcw> : 0-n = source window width <srch> : 0-n = source window height <dstx> : 0-n = destination window x-coordinate <dsty> : 0-n = destination window y-coordinate <dstw> : 0-n = destination window width <dsth> : 0-n = destination window height <bdr> : 0-1 = border state (off/on) <label> : string = window label PRESET_LIST=4:Preset_4:2:0:0:1200:1024:0:0:1280:1024:0:”Window_3” |
| PRESET_NAME=<entry>:<name> | Feedback on the name of a specific preset. <entry> : 1-10 = preset entry number <name> : string = preset name PRESET_NAME=5:Theater |
| PRIORITY=<input>:<value> | Feedback on the priority of the selected input’s window, or all input windows. Note: For the ‘ALL’ response, inputs are ordered according to their priority, i.e. the first input listed has priority 1, the second input listed has priority 2, and so on. <input> : 1-2 = selected input ALL = all inputs <value> : 1-2 = priority value PRIORITY=2:1 PRIORITY=ALL:2:1 |
| SET_MOVE=<x>:<y> | Reports the number of pixels and lines that the MOVE= command will use per step. <x> : 1-50 = number of pixels per left or right move <y> : 1-50 = number of lines per up or down move SET_MOVE=35:33 |
| SHARPNESS=<value>:<input> | Feedback on the current sharpness setting of the specified input. <value> : 0-100 = API sharpness range, in percent (%) -5..5 = device specific range <input> : 1-2 = selected input SHARPNESS=50:3 |
| TEST_PATTERN=<state> | Feedback on the current test pattern setting. <state> : 0 = Off 1 = Bars 2 = Horizontal Bars 3 = Grid 4 = Ramps 5 = DVI 6 = Snow 7 = Random TEST_PATTERN=3 |
| TIMING_LIST=<entry> :<name> :<hfp> :<hs> :<hbp> :<hact> :<vfp> :<vs> :<vbp> :<vact> :<hfreq> :<sync> :<hpol> :<vpol> :<il> | Feedback on the contents of the Input/Output Timing List. <entry> : 1-160 = List entry number <name> : string = Entry name <hfp> : 0-640 = Horizontal front porch <hs> : 16-640 = Horizontal sync <hbp> : 0-640 = Horizontal back porch <hact> : 16-1920 = Horizontal active <vfp> : 0-512 = Vertical front porch <vs> : 2-32 = Vertical sync <vbp> : 0-512 = Vertical back porch <vact> : 12-1440 = Vertical active <hfreq> : 12500-125000 = Horizontal frequency in Hz <sync> : 3-5 = Sync format <hpol> : 0-1 = Horizontal sync polarity <vpol> : 0-1 = Vertical sync polarity <il> : 0-1 = Interlaced/Noninterlaced TIMING_LIST=1:VESA_640x350_8:64:96:640:32:3:30:60:350:37861:5:1:0:0 |
| UNDER_SCAN=<input>:<state> | Feedback on the on/off state of the underscan functionality for the specified input. <input> : 1-2 = selected input <state> : OFF ON UNDER_SCAN=1:ON |
| VERSION=<value> | Reports the current version number of the NetLinx module. <value> : current version number in xx.yy format VERSION=1.06 |
| VIDEO=<input>:<state>[:<time>] | Feedback on the on/off state of the input’s video window. Note: Fade time is only returned by the Video= command, not the Video? query command. <input> : 1-2 = selected input <state> : 0 = Off 1 = On <time> : 5-50 = fade time VIDEO=2:1 VIDEO=1:1:50 |
| VIDEOMODE=<mode>:<input> | Feedback on the video format of the specified input. <mode> : NTSC PAL SXVGA <input> : 1-2 = selected input VIDEOMODE=2:1 |
| VIRTUAL_WINDOW=<x>:<y> | Feedback on how far the output window can be moved out of the output screen. <x> : 640-3200 = X direction multiplier <y> : 640-3200 = Y direction multiplier VIRTUAL_WINDOW=2000:3000 |

Table 2 - String Feedback Definitions

## Protocol Issues

- TestPattern command has a seventh option, Random, which is not documented.

- InputFormat command returns ‘SXVGA’ for RGB inputs, but this is not documented.

- VirtualScreenSize command is not documented, but works.

- Position command has a quit option, but this does not appear to be recognized by the firmware.

- Zoom command has a quit option, but this does not appear to be recognized by the firmware.

## Device Notes

- At startup or reboot, the DualView XL blasts the NetLinx with initialization info for about 60 seconds. It is recommended that you do not send any commands during this time as the response might be lost in the initialization info.

- Baud rates of 57600 and 115200 are supported by the device, however, the data received from the device at these rates is either indecipherable or invalid; therefore, these two baud rates are not supported in this module.

## Programming Notes

## Adding Functions to Modules

### Commands to the device

This module supplies a mechanism to allow additional device features to be added to software using the module.   This is the PASSTHRU command, which allows protocol strings to be passed through the module. The device-specific protocol must be known in order to use this feature.

As an example, suppose that a module for a projector has not implemented the 'white balance adjustment' feature.   The command that the projector protocol requires is 03H, 10H, 05H, 14H, followed by a checksum.   The documentation for the PASSTHRU command specifies that the module will automatically generate the checksum.   In this case, the following string should be sent from

the UI code to implement 'white balance adjustment'.

send_command vdvDevice,"'PASSTHRU=',$03,$10,$05,$14"

The reason to use PASSTHRU instead of sending a protocol string directly to the device port is that the device may require command queuing, calculation of checksums, or other internal processing, which would not be done if the string was sent directly. Because of this, it is best to filter all communication TO the device through the module. (The module documentation will indicate any processing that will be automatically done to the PASSTHRU string like checksum calculation.)

### Additional Feedback from the device

The module documentation indicates what feedback is provided.  If additional feedback is required, a CREATE_BUFFER for the device must be implemented in the user code to process the strings from the device manually. Note that the module will still be processing the response strings independently

and sending the interpreted feedback up to the user code.